OPERATION OF THE IMP

(from the article in Speaker Builder Magazine, copyright 1993,

Audio Amateur Publishing, Inc.)

(issue 3/93)

(describes use of IMP basic software; applies as well to IMP/M)

(figures not provided here; for further reference, see article)

In the first two parts, I described the hardware

construction and software operation for the IMP analyzer

system.  This beast is a versatile, but complex setup so this

last section will cover some step-by-step examples of

measurement procedures using the IMP.  If you have the

software program, but not an IMP module, you'll be able to

follow by loading up IMP data files provided on the

distribution disk.

These examples are far from exhaustive.  Variations such

as near/far field, fast/slow sample rate, SIZE changes,

selective echo inclusion, etc., will be left for the reader's

ingenuity and possible future articles on measurement

techniques.

All procedures follow a somewhat common form.  First,

the sensitivity of the PROBE2 input on the module is adjusted

for best use of the IMP's dynamic range.  Then a copy of the

test impulse as delivered by the amplifier is captured and

transferred to the computer, where it is transformed for use

as "cal", or correction data.  Then, if the measurement uses

the MIC input, the sensitivity of that is adjusted also

(PROBE2 shares a sensitivity control with PROBE1, and usually

need not be set separately).  The desired data is then

acquired from either the MIC or PROBE2, possibly edited, and

transformed with use of the cal data into the data

appropriate to the measurement.

INITIAL POWER-UP

When you first fire up the IMP module with the software,

you'll want to plug the MIC into the MIC jack and select

{Acquire, Repeat}.  The program will likely display a line

near the top of the plot window the first couple of times the

screen is drawn as the electrolytic capacitors in the circuit

settle down.  In about a minute, the plotted trace should

come to near the center of the window, and if you turn up V1,

you should be able to "see" your voice as you talk near the

MIC.

If the screen does not repeatedly plot, if the program

complains "module doesn't respond", or if the plot does not

resemble an typical oscilloscope plot of audio, something is

probably wrong.  Be sure the module power supply is plugged

in- the circuit can almost, but not quite, operate off the

power coming from the computer from the printer port line,

and will produce a very bizarre trace if you try to operate

it this way.  Also check for "solder bridges" on the board,

and that all ICs are properly installed.

ACOUSTIC FREQUENCY RESPONSE

Setup

Measurement of anechoic acoustic frequency response was

what initially prompted the design of the IMP, so that will

be covered first.  The equipment configuration for this

measurement is shown (again) in Figure 1.  Hook everything up

before applying power, and start out with the Preamp/Amp

volume control set to minimum.

If possible and practical, set the speaker so that its

tweeter/midrange radiation center is about equally spaced

between floor and ceiling and as far as poosible from side

walls or furniture.  You want the longest possible time lag

between the direct path to the microphone and the first

reflection off anything else.  The microphone is set at some

distance from the speaker (1 meter, or a little over 3 feet

is commonly used), and usually on the  tweeter axis.  The

variation of frequency response with microphone (i.e.,

listener) position is of course something you will want to

check out after you get the hang of the measurement process.

The probe connections should be made at the speaker

terminals.  Be very sure that the "hot" probe lead connects

to the "hot" lead as fed from the amplifier, and that the

"ground" probe lead connection corresponds to the amplifier

output ground.  Amplifiers, such as bridged amps, with

neither output lead at ground potential should NOT be used!

Amplifiers can sometimes respond unpleasantly to their

outputs being shorted, so be careful.

The plots used to illustrate this example are

measurements of an unmodified Dynaco A25 loudspeaker which I

borrowed for the purpose.  The only speakers I own which are

currently operational are tri-amped and are not typical of

what most speaker builders will be measuring.  If you will be

measuring bi, tri, or higher-amped systems, just connect the

probe leads to the output pulse signal from the IMP as it

feeds the preamp input.

OK, if the equipment is ready, turn on the power

switches.  Get the IMP software up and running, and verify

that the RATE (second screen line, second column) is set to

61.2kHz.  Press [F4] until the INPUT is set to PROBE1, which

is the "cal" probe.  From the top level menu, choose {Setup},

then {Cpu_speed} (in case you've forgotten, you do this by

pressing the capitalized letters in the desired selections,

in this case, "S" then "C").  You will be asked your

computer's "speed" in MHz.  Type in the clock speed of your

computer and press [ENTER].  Don't worry about

overestimating, as it will only affect data transfer time to

and from the IMP module.

Setting Input Levels

Now, get back to the top level menu by pressing "*",

then select {Acquire, Repeat}.  The screen will begin

repeatedly drawing the bottom plot.  All the while, test

pulses are being sent to your amplifier.  Turn up the volume

control until you can hear the "tic...tic..." sound of the

impulse from the speaker.  The choice of volume level is up

to you; too low and you will be covered under noise, too loud

and you may clip the amp (or even arc the speaker in the case

of electrostatics).  I generally set the volume so that the

impulses sound about as loud as a pebble being dropped into

an empty ceramic cup.  Variation of level is another good

area to investigate later.

If you now turn up the Probe Level control V2, you

should see the impulse from the amplifier output appear at

the left side of the plot on the lower half of the computer

screen.  Adjust the Probe Level control so that the peak of

the impulse is at about the 80% to 90% point of the plot

window height (see Figure 2; if the peak is negative, it

should be in the 10% to 20% range).  The control can move

fast, so use slight adjustments.  When you have it set, press

[esc] to end the "repeat" cycle.

Now, to get a cal sample, choose {*, Acquire, Collect}.

The system will collect and average five impulse responses.

When the process ends, press [F2] until SIZE is changed to

2048 (for a high resolution cal).  You can save this "cal"

response if you want by the sequence {*, File, Save,

Timeresp} from the top menu.  The cal impulse for the A25

measurement is on the IMP software distribution disk under

the name "DYNACAL.IMP".  In case you want to follow along

with the IMP software on your computer and don't have a

completed IMP module, you can load this cal in (and other

example files referred to hereafter) by the sequence {*,

File, Retreive}, and answer "DYNACAL" (or the indicated name)

when prompted for a file name.

The cal impulse data must be transformed to cal

frequency response data.  To transform all the data, make

sure that MKR1, as shown on the second screen line, is set to

sample number 1 and MKR2 is set to the same value as SIZE

(2048 in this example; use function keys [F5] and [F6] to set

these values).  Press "*" to get back to the top level (if

not there already) and select {Transform, Fft}.  After the

little clock has finished going around, a rather tame

frequency response magnitude plot will be plotted at screen

bottom.  This is your cal response.  To so declare it, select

{Set_cal} and answer "Y" to the neurotic question that

the program will ask.  A little "c" will appear to the left

of the IMP logo in the top screen line to indicate that cal

data is in memory.

It's time to get a speaker measurement:  Press [F4]

until the INPUT selected is MIC.  Leave the amp volume

control alone and select {Acquire, Repeat} again.  This time

you will set the level of the MIC Level control V1 until the

displayed perturbation peaks again near 80% to 90% (or 10% to

20%) from the bottom of the lower window, then hit [ESC].

Because of the finite speed of sound as the impulse travels

from speaker to microphone, this peak will not be at the far

left side of the screen.  The wiggle you see is how the

speaker renders that pulse you saw earlier in the acquired

cal.  To get a cleaner, averaged copy, now select {Acquire,

Collect}.  After the 15 or so samples have been taken and the

tick sounds have stopped, you may want to save this data to

disk, too.  1024 points is probably more than adequate, so

press [F2] until SIZE is 1024 before the {*, File, Save,

Time_file} sequence.  Press [F1] to redraw the display with

the new setting.  My data for the A25 at this point is in the

file "DYNACOUS.IMP" if you can't collect your own.

Frequency Response Curve

Now, for the good part.  Press [F5] once and use the

left/right arrow keys to adjust MKR1 to just to the left of

the beginning of the perturbation.  Holding down the control

keys will make the marker move faster if you have a long way

to go.  When you have it there, press [ENTER] to seal it, and

notice that the top display is updated.  Press [F6] and

likewise put MKR2 to just before the first echo reflection

wiggle.  This condition is shown for my A25 data in Figure 3.

To turn this edited impulse response into a frequency

response, select {*, Transform, Fft}.  Wait a bit and a curve

should appear.  This is the raw frequency response-- it is

still affected by amplifier and IMP filter responses.  Select

{Cal} and the high end should rise a bit (and overall level

will generally change).

Let's spend some time on the presentation of this plot.

If you move the markers around now, you will notice that the

frequencies and dB values are read out on-screen.  Put MKR1

at 1kHz (press [F5], type in "1K" and [ENTER]) and MKR2 at

15kHz.  By the way, don't take all those significant digits

in the readouts seriously.  You won't likely find anything

like 0.001 dB precision very repeatable.

The starting frequency of the plot probably isn't

optimal; press [SHIFT] and [F5] together, then type in "500"

when prompted for the starting frequency.  [F8] can be used

to raise or lower the curve on the grid--negative values make

it go down.  [SHIFT] and [F8] together can be used to change

the dB scale, if desired.  A 20 dB/division scale will make

any speaker look great!

Lastly, you may want to see phase response along with

the magnitude.  Just press [F10], then select

{Freq_resp}.  Note that the phase is relative to the

reference plane determined by the original time-domain MKR1

setting!  You can modify this curve by use of the [F9] delay

adjustment key.  A reasonably formatted picture of the A25's

response is shown in Figure 4.

And Now, Fast Frequency Response

That may seem like a lot of effort, and so it may be the

first time.  Before long, however, you'll go through all this

setup in minutes without even thinking about it.  And after

you have it all set up, doing it again will take hardly any

time at all.

Remember the Auto_measure facility?  To get another

response after, say, making a crossover tweak or changing the

microphone height, just use the sequence {*, auto_Measure,

Acoustic} and let the computer run through the collection,

calibrating and transforming itself while you consider your

next move.  Selection of whether to use a cal and whether a

new one is to be obtained each time can be done via {*,

auto_Measure, Setup}.  Note that time marker positions may

need moving if the effective microphone distance changes much

-use [F1] to get to the proper display in which to move them.

Transformations of near field measurements, of course,

should use the time markers set at 1 and SIZE.

Some Other Trasformations

Waterfall plots are easy to do at this point.  If gain,

scale and frequency range are set to your liking, just select

{*, Transform, Waterfall}.  You will be asked whether or not

you want cal data to be included (usually, the answer will be

"1" for "yes").  A waterfall plot made from the A25 is shown

in Figure 5.  If your computer is slow or if you are

impatient, you may want to set SIZE to a smaller value before

doing a waterfall.  These plots can take quite a while on

5MHz PCs!

You can also transform the "cal'd" anechoic frequency

response data back to corrected time-domain impulse response

data.  First, if you just plotted a waterfall, you need to

get frequency data back as it has been wiped clean.  Select

{*, Transform, Fft, Cal}.  Be sure to set delay (via [F9])

back to 0.  Then select {*, Transform, Ifft} and the time

data will now be corrected for the amp and IMP filters.  I

have noticed some fuzziness on many Ifft'd curves, which

seems to have something to do with a response zero in the cal

data near 31kHz at the Nyquist frequency.

ELECTRICAL RESPONSE OF CROSSOVER

Figure 6 shows the equipment setup for measuring

electrical frequency response of a crossover, in this

example for the Dynaco A25's tweeter.  This is a rather

unexciting first-order network, but the procedure still holds

for more exotic circuits.  Sample rate used here was 61.2kHz.

Acquisition of the cal is the same as for the acoustic

measurement.  Do be sure that the cal is transformed from

data of SIZE at least as large as that desired from the

tweeter probe, or the software will complain that the cal is

not compatible.  On the distribution disk, "DYNACAL.IMP" can

again be used as cal for this example.

When probe level has been set and the cal acquired,

transformed and Set_cal has been done, just press [F4] until

PROBE2 is selected as INPUT.  Do NOT readjust the probe level

control V2.  Select the sequence {*, Acquire, Collect} and

wait for the acquisition to complete (file "DYNATWEE.IMP").

Then, first making sure that the markers are placed at the

beginning and end of the data window, select {*, Transform,

Fft, Cal}.

When the calculations are completed, the crossover

response curve will be presented.  You can format the display

for taste and clarity as described previously, but if you

have the GAIN set to 0 dB, you will have a plot of actual

rather than relative loss of the network.  Phase data can be

displayed, again, via [F10] followed by {Freq_resp}.  The

Dynaco A25's tweeter crossover curve, such as it is, is shown

in Figure 7.

If you look in the auto_Measure submenu, you will find

an automated procedure for electrical response measurement

which may be useful when tweaking a network.

IMPEDANCE MEASUREMENT WITH THE IMP

For this example, refer to Figure 8.  The resistor R is

selected to be a known value of resistance near the nominal

impedance of the speaker.  Results will be most accurate for

measured data near in value to this resistance.  A 10 ohm

resistor is easy to come by, or Radio Shack sells 8 ohm, 5%

power resistors which work well.

My example is for the impedance of a woofer in a home-

made ported system.  We are interested in data at from

a few Hz to under 500 Hz, so the rate should be set (via

[F3]) to 1.92kHz.  You must tell the computer what value of

resistor you are using by executing the sequence {*, Display,

Format, Scale, Ref_resistor} and answering the prompt.  The

best resolution will result if you use a SIZE of 4096, but

this is usually only needed if investigating very low

frequency stuff.

Set the INPUT to PROBE1 via the [F4] key.  Turn on all

equipment and select {*, Acquire, Repeat}.  Turn the preamp

volume control up until a "pop" sound is heard about as loud

as a moderate door knock.  Adjust the probe level control for

the usual 80%-90% or 10%-20% of full scale display for the

peak of the impulse.  This can be confusing, as each sample

is several seconds long and the display lags significantly

behind the adjustment--go slow and be patient.  When you have

it OK, hit [ESC].

Now make sure that MKR1 is set to 1 (use [F5] if it

isn't) and that MKR2 is at SIZE (use [F6] to set it to the

same value as shown under SIZE, 4096 for the example).

Select {*, Acquire, Collect} and wait for the cal run to

complete.  Save this time data to disk, if you want to look

at or print the impedance plot out in a later session; cal

data for the example can be retreived from the file

"TWWCAL.IMP".  Finish the cal process by transforming and

declaring the cal: {*, Transform, Fft, Set_cal}.

You can set the frequency range now, if you wish.  Use

[SHIFT F5] and [SHIFT F6] to set the lower value to 10 Hz and

the upper value to 700 Hz.  Now, set INPUT to PROBE2 using

[F4] and without disturbing any controls, execute {*,

Acquire, Collect} to get the data at the speaker side of the

resistor.  The example data for this is on the distribution

disk under the name "TWWZ.IMP".

To compute these two data sets into a complex impedance

curve, use {*, Transform, calcZ}.  Results using the example

files are shown in Figure 9 (the markers are set to pinpoint

the box resonance).  The Scale can be adjusted using {*,

Display, Format, Scale, Ohms/div}.  An automatic routine for

impedance measurement also exists under the auto_Measure

submenu.  If you want to repeat the impedance plot later via

disk storage, however, you'll have to collect and save the

cal data the slower way.

IMPULSE DATA FILE FORMAT

If you load up one of the "?.IMP" data files into an

ASCII text editor, you'll see it is only a list of numbers

saved one to a line.  If you want to to generate your own

artificial impulse responses to analyze with the IMP

software or use the IMP data in other graphing programs, here

is a quick rundown of the format.

The first number is the SIZE of the saved data, as was

current when the data was saved.  The second number is the

number of valid data points, which may be less than SIZE if

the data was from an IFFT operation.  After this comes the

sample rate, which the IMP program will round to the nearest

of 1.9kHz and 66.2kHz, so don't bother to try others.  Next

comes either the value 1 or 0; 1 means the data is corrected

(via an IFFT of a cal'd response), 0 means it isn't.  After

this come the data points, the number of them being equal to

SIZE.  And that's it.

ACKNOWLEDGEMENTS

I'd like to thank first and foremost my wife Carol

Stewart who proofed the early drafts, did camera work,

corrected and clarified the sentences, put up with a project

which grew (in involvement) far beyond what I expected and

what I led her to expect, and provided much needed support.

My three children deserve thanks for putting up with my long

hours at the keyboard.  Tom Alverson was most helpful in all

matters relating to computer systems.  Gary Smith transfered

the laser plots of the PC Board patterns.  Ray Andrews

provided speakers and drivers to test as well as general

inspiration in the art of speaker building and audio.

ABOUT THE AUTHOR

Bill Waslo is an Electrical Engineer for a midwestern

electronics firm and is involved with the design of RF

filters and analog signal processing.  He is a graduate of

the University of Cincinnati and has been involved with

electronics since he was allowed to hold a soldering iron.

Besides music and audio, his interests are his family,

organic gardening and mystery novels.